# ECA5610-  Advance wireless communication systems

Name: S.Gokul

Reg no:192512373

Experiment Number: 1

Simulation and Comparative Analysis of FDMA, TDMA and CDMA:

Resource Allocation, User Capacity and Mobile-Generation Context

Aim:

To simulate and compare the basic resource-allocation methods used in FDMA, TDMA and CDMA, and examine how their assumed user capacity and interference behaviour differ.

# Objectives:

- Model how FDMA, TDMA and CDMA allocate communication resources among users.

- Estimate and compare user capacity under stated assumptions.

- Plot how estimated CDMA SIR changes as the number of simultaneous users increases.

- Relate the access methods to representative mobile generations and interpret the limits of the simplified model.

# Requirements:

  - Computer with MATLAB

  - No hardware is required

# Program : for objective 1,2

% Comparison of FDMA, TDMA and CDMA

% Simplified illustrative model clc;

clear; close all;

% Illustrative system parameters

totalBandwidthHz = 1e6; % Total bandwidth: 1 MHz fdmaChannelHz = 200e3;	% FDMA channel width: 200 kHz tdmaSlots = 8;	% Number of TDMA slots per frame processingGain = 64;		% CDMA processing gain targetSIRdB = 7;	% Minimum acceptable CDMA SIR

% Capacity estimation

fdmaUsers = floor(totalBandwidthHz / fdmaChannelHz); tdmaUsers = tdmaSlots;

targetSIR = 10^(targetSIRdB / 10);

cdmaUsers = floor(1 + processingGain / targetSIR);

% Display estimated user capacity fprintf('Illustrative Maximum Supported Users\n'); fprintf('FDMA: %d users\n', fdmaUsers); fprintf('TDMA: %d users\n', tdmaUsers);

fprintf('CDMA: %d users at a target SIR of %.1f dB\n\n', ... cdmaUsers, targetSIRdB);

% Calculate CDMA SIR for different user counts K = 2:30;

sirLinear = processingGain ./ (K - 1); sirDb = 10 * log10(sirLinear);

% Create comparison table

method = {'FDMA'; 'TDMA'; 'CDMA'};

supportedUsers = [fdmaUsers; tdmaUsers; cdmaUsers]; results = table(method, supportedUsers);

disp(results);

% Plot 1: Estimated user capacity figure;

bar(categorical(method), supportedUsers); ylabel('Maximum Supported Users');

title('FDMA, TDMA and CDMA Capacity Comparison'); grid on;

% Plot 2: CDMA SIR versus number of users figure;

plot(K, sirDb, 'bo-', 'LineWidth', 1.5); hold on;

yline(targetSIRdB, '--r', 'Target SIR') xlabel('Number of Simultaneous CDMA Users (K)'); ylabel('Estimated SIR (dB)');

title('CDMA SIR versus Number of Users'); legend('Estimated SIR', 'Target SIR', 'Location', 'best'); grid on;

hold off;

# Output:

+

OBJECTIVE 3:

Program :

generation = ["1G"; "2G"; "2G"; "3G"; "4G"; "5G"];

system = ["AMPS"; "GSM"; "IS-95"; "UMTS"; "LTE"; "5G NR"];

accessMethod = ["FDMA"; "TDMA"; "CDMA"; "WCDMA"; "OFDMA"; "OFDMA-based"];

keyIdea = ["Separate frequency channels"; ... "Separate time slots"; ...

"Code-based user separation"; ... "Wideband code-based access"; ... "Orthogonal frequency resources"; ... "Flexible OFDM-based radio access"];

evolutionTable = table(generation, system, accessMethod, keyIdea); disp(evolutionTable);

# OUTPUT:

OBJECTIVE 4:

Program:

figure; tiledlayout(1,3);

% FDMA: users occupy different frequency channels nexttile;

hold on;

colors = lines(4); for u = 1:4

rectangle('Position',[0, u-1, 1, 0.8], ...

'FaceColor',colors(u,:), 'EdgeColor','k');

text(0.5, u-0.6, sprintf('User %d',u), ... 'HorizontalAlignment','center');

end

xlim([0 1]); ylim([0 4]); xlabel('Time');

ylabel('Frequency channel'); title('FDMA'); set(gca,'YTick',[]);

hold off;

% TDMA: users occupy different time slots on one frequency channel nexttile;

hold on; for u = 1:4

rectangle('Position',[u-1, 0, 0.8, 1], ...

'FaceColor',colors(u,:), 'EdgeColor','k');

text(u-0.6, 0.5, sprintf('User %d',u), ... 'HorizontalAlignment','center');

end

xlim([0 4]); ylim([0 1]); xlabel('Time slot'); ylabel('Frequency channel'); title('TDMA'); set(gca,'YTick',[]);

hold off;

% CDMA: users share the same time and frequency band using different codes nexttile;

hold on; for u = 1:4

rectangle('Position',[0, 0, 1, 1], ...

'FaceColor',colors(u,:), 'FaceAlpha',0.22, ...

'EdgeColor',colors(u,:), 'LineWidth',1.5);

end

text(0.5, 0.62, 'Users 1-4 share', 'HorizontalAlignment','center'); text(0.5, 0.40, 'band; different codes', 'HorizontalAlignment','center'); xlim([0 1]); ylim([0 1]);

xlabel('Time'); ylabel('Frequency band'); title('CDMA'); set(gca,'YTick',[]);

hold off;

sgtitle('Conceptual Time-Frequency Resource Allocation');

# Output :

Result:

Thus, FDMA, TDMA and CDMA resource allocation techniques were studied and compared based on user capacity and signal-to-interference ratio.